OPERATION OF THE NANOPURE WATER PURIFICATION SYSTEM

Joseph W. Stucki

University of Illinois

Specifications

Barnstead Fisher NanoPure System Model D4741 (four-cartridge system)

Input is distilled H2O

Cartridges

Fisher 09-050-276 (D-5027)	UltraPure DI Cartridge, no spring

Fisher 09-034 (D-0803)	Demineralizing Cartridge Blue

Fisher 09-034-5C (D-0836)`	Pretreatment Cartridge

Fisher 09050-267 (D-5021)	Organic Free Cartridge Reverse Flow

Operation

1.	Be sure inlet flow valve is open.

2.	Using the electronic control pad, press to the “On” position and allow it to run for a few seconds. The reading should be 18 or greater (a reading of “ERR” is ok – indicates that purity has surpassed 18 mOhm cm-1).

3.	Return setting to “SBY” or “Off (blank)” positions when finished using the water.

4.	The final in-line filter is not used.

5.	Water can be transferred to lab W-324 by opening the white valve in the grey PVC line above the electronic pad.

6.	When reading drops below 18 mOhm cm-1, change the cartridges (cost is about $350).